Networking Training Program

Module 05 — Layer 3 Assignment

Network Layer (Layer 3) Concepts & Practicals

# Q1. ARP Packet Analysis using Wireshark

## Theory

ARP (Address Resolution Protocol) maps an IP address to its corresponding MAC address within a local network. When a device wants to communicate with another, it sends an ARP request as a broadcast asking 'Who has this IP?' The device with that IP replies with its MAC address. Wireshark lets us capture and study these frames in real time.

## Key Fields in ARP Packets

| Field | ARP Request | ARP Reply |
| --- | --- | --- |
| Opcode | 1 (Request) | 2 (Reply) |
| Sender IP | 192.168.1.10 (Requester) | 192.168.1.1 (Target) |
| Sender MAC | Real MAC of sender | Real MAC of responder |
| Target IP | IP being asked about | IP of original requester |
| Target MAC | 00:00:00:00:00:00 (Unknown) | Filled with requester's MAC |

## Practical Steps

- Open Wireshark and select the active network interface (e.g., Ethernet or Wi-Fi).

- In the filter bar, type arp and press Enter to capture only ARP packets.

- From another terminal, ping a device on the same LAN (e.g., ping 192.168.1.1).

- You will see ARP Request and ARP Reply packets in Wireshark.

- Click on the ARP Request — check the Sender IP and Target IP fields.

- Click on the ARP Reply — note the Sender MAC now maps to the Target IP.

Note: The sender's IP and MAC are included in both request and reply. The target MAC is empty (00:00:00:00:00:00) in the request, and filled in by the responding device in the reply.

## Screenshot — Wireshark ARP Capture

Figure 1: Wireshark capturing ARP Request (packet #2, highlighted) and ARP Reply packets with packet detail panel showing sender/target IP and MAC fields

# Q2. ARP Spoofing Attack Simulation — Packet Tracer

## Theory

ARP Spoofing is an attack where a malicious device sends fake ARP replies to link its MAC address to a legitimate IP. This causes traffic meant for a real device to be routed through the attacker — a classic Man-in-the-Middle (MITM) attack.

## Simulation Steps in Cisco Packet Tracer

- Create a simple topology: PC-A, PC-B, and Attacker PC connected to a switch.

- Assign IPs: PC-A: 192.168.1.10, PC-B: 192.168.1.20, Attacker: 192.168.1.30.

- PC-A pings PC-B — observe normal ARP behavior using PDU simulation mode.

- Now simulate the attacker sending fake ARP: Attacker claims its MAC is for 192.168.1.10.

- PC-B now sends traffic to 192.168.1.10 but it reaches the Attacker instead.

- Use 'ARP -a' on the PCs to see the poisoned ARP cache entries.

## Effect on Network Devices

| Device | Expected Behavior | After ARP Spoof |
| --- | --- | --- |
| PC-A | Communicates normally with PC-B | May receive redirected traffic |
| PC-B | Sends packets to PC-A's real MAC | Sends to attacker's MAC instead |
| Attacker | Not involved in original traffic | Receives and can intercept packets |

Defense: Enable Dynamic ARP Inspection (DAI) on managed switches to prevent ARP spoofing.

## Screenshot — ARP Spoofing Topology

Figure 2: Cisco Packet Tracer showing ARP spoofing attack — attacker sending fake ARP reply, PC-B ARP cache poisoned with attacker MAC, MITM established

# Q3. Manually Configuring Static IPs on Client Devices

## Theory

Static IP configuration means assigning a fixed IP address to a device manually instead of letting DHCP assign one automatically. This is useful for servers, printers, or any device that needs a permanent address. A correctly configured device should be reachable via ping from other devices on the same subnet.

## On Windows PC — Steps

- Go to Control Panel > Network and Internet > Network Connections.

- Right-click on your adapter > Properties > select IPv4 > Properties.

- Choose 'Use the following IP address' and enter: IP: 192.168.1.50, Subnet Mask: 255.255.255.0, Gateway: 192.168.1.1.

- Click OK. Open CMD and type: ipconfig to verify.

- Then type: ping 192.168.1.51 to test connectivity with another static device.

## Screenshot — Static IP Configuration and Ping Test

Figure 3: Windows IPv4 Properties dialog with static IP configured (192.168.1.50) and CMD showing ipconfig output and successful ping responses from 192.168.1.51

# Q4. Capturing DHCP Messages using Wireshark

## Theory

DHCP (Dynamic Host Configuration Protocol) automatically assigns IP addresses to devices on a network. The process involves 4 steps known as DORA: Discover, Offer, Request, and Acknowledge.

| Step | Type | Direction | Purpose |
| --- | --- | --- | --- |
| 1 | Discover | Client → Broadcast | Client looks for any DHCP server |
| 2 | Offer | Server → Client | Server offers an available IP address |
| 3 | Request | Client → Broadcast | Client formally requests the offered IP |
| 4 | Acknowledge | Server → Client | Server confirms the lease of the IP |

## Practical Steps

- Open Wireshark and select your active network interface.

- In the filter bar, type: bootp to capture DHCP traffic.

- Open CMD and run: ipconfig /release to release current IP.

- Then run: ipconfig /renew to trigger a fresh DHCP cycle.

- Wireshark will capture all 4 DHCP messages — Discover, Offer, Request, Acknowledge.

- Click each message and inspect: Transaction ID, Client IP, Your IP, DHCP Option fields.

Tip: DHCP uses UDP ports 67 (server) and 68 (client). Both Discover and Request are broadcast messages since the client has no IP yet.

## Screenshot — DHCP DORA Capture

Figure 4: Wireshark showing all 4 DHCP messages (Discover/Offer/Request/ACK) color-coded with packet detail panel expanded for the Discover message showing Transaction ID and options

# Q5. Subnetting 192.168.1.0/24 into 4 Subnets

## Theory

Subnetting divides a large network into smaller sub-networks. To create 4 subnets from a /24 network, we need 2 extra bits (2² = 4), making the new prefix /26. Each subnet will have 64 addresses (62 usable hosts).

## Subnet Calculation

| Subnet | Network Address | Subnet Mask | Host Range | Broadcast |
| --- | --- | --- | --- | --- |
| Subnet 1 | 192.168.1.0 | 255.255.255.192 (/26) | 192.168.1.1 – 192.168.1.62 | 192.168.1.63 |
| Subnet 2 | 192.168.1.64 | 255.255.255.192 (/26) | 192.168.1.65 – 192.168.1.126 | 192.168.1.127 |
| Subnet 3 | 192.168.1.128 | 255.255.255.192 (/26) | 192.168.1.129 – 192.168.1.190 | 192.168.1.191 |
| Subnet 4 | 192.168.1.192 | 255.255.255.192 (/26) | 192.168.1.193 – 192.168.1.254 | 192.168.1.255 |

## Cisco Packet Tracer Setup

- Open Packet Tracer. Add 1 Router and 4 PCs (one for each subnet).

- Connect each PC to the router using appropriate interfaces.

- Assign IPs on PCs using the first usable IP of each subnet.

- Configure router interfaces with the gateway IP of each subnet.

- Ping from PC in Subnet 1 to PC in Subnet 2 to verify routing is working.

## Screenshot — Subnetting Topology and Ping Test

Figure 5: Cisco Packet Tracer topology showing router connected to all 4 /26 subnets with ping test output confirming 100% success rate across all subnets

# Q6. IP Address Classification — Classes A, B, and C

## Theory

IP addresses are divided into classes based on their first octet. Each class has a default subnet mask and covers a specific range of addresses. Private IP addresses within these classes are used for internal networks and are not routed on the internet.

## Given IP Addresses Analysis

| IP Address | First Octet | Class | Default Subnet Mask | Reason |
| --- | --- | --- | --- | --- |
| 10.1.1.1 | 10 | Class A | 255.0.0.0 (/8) | 1–126 = Class A |
| 172.16.5.10 | 172 | Class B | 255.255.0.0 (/16) | 128–191 = Class B |
| 192.168.1.5 | 192 | Class C | 255.255.255.0 (/24) | 192–223 = Class C |

## Screenshot — IP Class Identification

Figure 6: IP address classification showing all 3 given IPs identified with their class, default subnet mask, private range, and complete class reference table

Note: 127.x.x.x is reserved for loopback testing. 224–255 are Class D/E and not used for regular hosts.

# Q7. NAT Configuration in Cisco Packet Tracer

## Theory

NAT (Network Address Translation) allows multiple devices using private IP addresses to access the internet through a single public IP address. The router translates the source private IP to its public IP before forwarding packets to the internet — and reverses this on the way back.

## Network Setup

| Device | Interface | IP Address | Type |
| --- | --- | --- | --- |
| PC1 | Ethernet | 192.168.1.10 | Private |
| PC2 | Ethernet | 192.168.1.11 | Private |
| Router | GigabitEthernet0/0 (inside) | 192.168.1.1 | Private Gateway |
| Router | GigabitEthernet0/1 (outside) | 203.0.113.1 | Public IP |
| External Server | Any interface | 8.8.8.8 | Public (simulated) |

## Router Configuration Commands

Router(config)# ip nat inside source list 1 interface g0/1 overload

Router(config)# access-list 1 permit 192.168.1.0 0.0.0.255

Router(config)# interface g0/0

Router(config-if)# ip nat inside

Router(config-if)# interface g0/1

Router(config-if)# ip nat outside

## Before and After NAT

| Stage | Source IP | Destination IP | Explanation |
| --- | --- | --- | --- |
| Before NAT (inside) | 192.168.1.10 | 8.8.8.8 | Private IP sent by PC |
| After NAT (outside) | 203.0.113.1 | 8.8.8.8 | Router replaces with public IP |
| Reply (coming back) | 8.8.8.8 | 203.0.113.1 | Reply reaches router's public IP |
| After reverse NAT | 8.8.8.8 | 192.168.1.10 | Router translates back to private IP |

The key observation: before NAT, packets carry the private IP (192.168.1.10). After NAT at the router, the source IP becomes the public IP (203.0.113.1). This is how multiple devices share one public IP.

## Screenshot — NAT Configuration and Packet Capture

Figure 7: Cisco Packet Tracer NAT topology showing private-to-public IP translation, Wireshark packet capture comparing source IPs before and after NAT, and NAT translation table from show ip nat translations command