1) Test-Connection confirms whether the destination is reachable (similar to ping).

nslookup resolves domain names into IP addresses using DNS servers.

🔹 Observations:

- Test-Connection confirms whether the destination is reachable (similar to ping).

- nslookup resolves domain names into IP addresses using DNS servers.

2) Wireshark Analysis (DNS, TCP, UDP)

Objective:

To capture and analyze network packets.

🔹 Steps:

- Open Wireshark

- Select active network interface

- Start capture

- Apply filters:

  - DNS → dns

  - TCP → tcp

  - UDP → udp

🔹 Observations:

- DNS: Query and response packets

- TCP: 3-way handshake (SYN, SYN-ACK, ACK)

- UDP: Connectionless communication

- Packet headers contain:

  - Source IP

  - Destination IP

  - Protocol

  - Flags

For TCP :

3)  Traceroute / Tracert Analysis

Objective:

To track the path taken by packets.

Command:

tracert www.google.com

🔹 Observations:

- Displays intermediate routers (hops)

- Shows latency at each hop

- Helps identify network delays or failures

Output :

For Google and Facebook

For Linux

Common Options in traceroute (Linux)

1. -n → No DNS resolution

traceroute -n www.google.com

2. -m → Max hops

traceroute -m 15 www.google.com

3. -w → Wait time

traceroute -w 2 www.google.com

4. -q → Number of probes per hop

traceroute -q 3 www.google.com

5. -i → Specify interface

traceroute -i eth0 www.google.com

6. -T → Use TCP instead of UDP

traceroute -T www.google.com

4) Objective:

To simulate a network environment.

Steps:

- Open Cisco Packet Tracer

- Add:

  - 2 PCs

  - 1 Switch

- Connect using straight-through cable

- Assign IP addresses

- Test using ping

Result:

- Successful communication between PCs

Output:

7. Configure Management VLAN with Remote Access (SSH/Telnet)

Steps:

- Create VLAN:

Switch(config)# vlan 99
Switch(config-vlan)# name Management

- Assign IP to VLAN interface:

Switch(config)# interface vlan 99
Switch(config-if)# ip address 192.168.99.1 255.255.255.0
Switch(config-if)# no shutdown

- Configure default gateway:

Switch(config)# ip default-gateway 192.168.99.254

4a)         Telnet:

Switch(config)# line vty 0 4
Switch(config-line)# password 1234
Switch(config-line)# login

Testing

Use : telnet ip

4b)       Enable SSH:

Switch(config)# hostname Switch
Switch(config)# ip domain-name lab.com
Switch(config)# crypto key generate rsa
Switch(config)# username admin password 1234
Switch(config)# line vty 0 4
Switch(config-line)# login local
Switch(config-line)# transport input ssh telnet

Testing:

- Use: ssh admin@192.168.99.1

Outputs :

Q8 – Voice VLAN (20) & Data VLAN (10)

Step 1: Enter Configuration Mode

Switch> enable
Switch# configure terminal

Step 2: Create VLAN 10 (Data)

Switch(config)# vlan 10
Switch(config-vlan)# name DATA
Switch(config-vlan)# exit

Step 3: Create VLAN 20 (Voice)

Switch(config)# vlan 20
Switch(config-vlan)# name VOICE
Switch(config-vlan)# exit

Step 4: Configure Switch Port

Switch(config)# interface fa0/1
Switch(config-if)# switchport mode access
Switch(config-if)# switchport access vlan 10
Switch(config-if)# switchport voice vlan 20
Switch(config-if)# no shutdown

Step 5: Connect Devices

- Connect Switch Fa0/1 → IP Phone

- Connect PC → IP Phone (PC port)

Step 6: Configure IP Address

- PC → IP from VLAN 10 network (e.g., 192.168.10.1)

- IP Phone → IP from VLAN 20 network (via DHCP or manual)

Step 7: Verify Configuration

Switch# show vlan brief
Switch# show interfaces fa0/1 switchport

Step 8: Result

- PC traffic → VLAN 10

- IP Phone traffic → VLAN 20

- Both work on same port successfully

9. VLAN 10 cannot communicate with VLAN 20 (Troubleshooting)

Reason:

- VLANs are isolated (no inter-VLAN routing)

Solution:

- Configure router or Layer 3 switch

- Enable inter-VLAN routing

Example (Router-on-a-Stick):

Router(config)# interface g0/0.10
Router(config-subif)# encapsulation dot1Q 10
Router(config-subif)# ip address 192.168.10.1 255.255.255.0

Router(config)# interface g0/0.20
Router(config-subif)# encapsulation dot1Q 20
Router(config-subif)# ip address 192.168.20.1 255.255.255.0

10) Inter- Vlan communication

11)  Implementation of Extended Access Control List

Step 1: Enter Privileged and Global Configuration Mode

Access the router CLI and enter privileged mode, followed by global configuration mode using:

- enable

- configure terminal

Step 2: Create an Extended Access Control List

Define an extended ACL to control traffic based on protocol and port number.

- Permit HTTP traffic (port 80):

  - access-list 101 permit tcp any any eq 80

- Deny all other traffic:

  - access-list 101 deny ip any any

Step 3: Apply the ACL to an Interface

Apply the ACL to the appropriate interface using the ip access-group command.

- In a router-on-a-stick configuration, apply it on the subinterface:

  - interface fa0/0.10

  - ip access-group 101 in

Step 4: Verify the ACL Configuration

Use verification commands to check whether the ACL is configured and active:

- show access-lists

Step 5: Test the Configuration

- Attempt to send ICMP (ping) packets between devices. These should be blocked.

- Attempt HTTP communication (if a server is present). This should be allowed.

OUTPUT:

12. Configure a Standard Access Control List (ACL)

Objective:

Allow traffic only from a specific IP range and deny all others.

Steps:

- Enter Privileged Mode

Router> enable

- Enter Global Configuration Mode

Router# configure terminal

- Create a Standard ACL

- Example: Permit network 192.168.1.0/24

Router(config)# access-list 10 permit 192.168.1.0 0.0.0.255

- Apply ACL to Interface

- Apply inbound on interface (e.g., FastEthernet0/0)

Router(config)# interface fa0/0
Router(config-if)# ip access-group 10 in

- Exit Configuration Mode

Router(config-if)# exit
Router(config)# exit

Testing:

- Test Connectivity

- From allowed IP → Should work

- From other IP → Should be blocked

- Verify ACL

Router# show access-lists

13. Configure an Extended ACL (Block HTTP/FTP Traffic)

Objective:

Block specific applications like HTTP (port 80) and FTP (port 21).

Steps:

- Enter Configuration Mode

Router# configure terminal

- Create Extended ACL

Router(config)# access-list 101 deny tcp any any eq 80
Router(config)# access-list 101 deny tcp any any eq 21
Router(config)# access-list 101 permit ip any any

- Apply ACL to Interface

Router(config)# interface fa0/0
Router(config-if)# ip access-group 101 in

- Exit

Router(config-if)# exit
Router(config)# exit

Testing:

- Test HTTP/FTP Access

- Open website → Should fail

- Other traffic (ping, etc.) → Should work

- Verify

Router# show access-lists

Outputs :

14. Configure NAT (Static, Dynamic, PAT)

A. Static NAT

Objective:

Map one private IP to one public IP.

Steps:

- Define inside and outside interfaces:

Router(config)# interface fa0/0
Router(config-if)# ip nat inside

Router(config)# interface fa0/1
Router(config-if)# ip nat outside

- Configure static mapping:

Router(config)# ip nat inside source static 192.168.1.2 200.1.1.2

- Verify:

Router# show ip nat translations

B. Dynamic NAT

Objective:

Map private IPs to a pool of public IPs.

Steps:

- Create ACL:

Router(config)# access-list 1 permit 192.168.1.0 0.0.0.255

- Define NAT pool:

Router(config)# ip nat pool MYPOOL 200.1.1.1 200.1.1.10 netmask 255.255.255.0

- Bind ACL to pool:

Router(config)# ip nat inside source list 1 pool MYPOOL

- Verify:

Router# show ip nat translations

C. PAT (Port Address Translation)

Objective:

Multiple devices share one public IP.

Steps:

- Create ACL:

Router(config)# access-list 1 permit 192.168.1.0 0.0.0.255

- Enable PAT:

Router(config)# ip nat inside source list 1 interface fa0/1 overload

- Verify:

Router# show ip nat translations

15. Network Performance Analysis using iPerf (Linux)

Objective:

To measure network performance parameters such as bandwidth, jitter, delay, and packet loss using the iPerf tool in a Linux environment.

Procedure:

Step 1: Install iPerf3

Open terminal and install iPerf3 using:

sudo apt update
sudo apt install iperf3 -y

Step 2: Start iPerf Server

In the first terminal, run:

iperf3 -s

This system acts as the server.

Step 3: Run iPerf Client

In another terminal, execute:

iperf3 -c 127.0.0.1

This connects the client to the server using localhost.

Step 4: Perform Different Tests

1. TCP Test

iperf3 -c 127.0.0.1

- Measures bandwidth and throughput

2. UDP Test

iperf3 -c 127.0.0.1 -u

- Measures jitter and packet loss

3. Reverse Mode

iperf3 -c 127.0.0.1 -R

- Tests data transfer from server to client

4. Parallel Streams

iperf3 -c 127.0.0.1 -P 5

- Simulates multiple simultaneous connections

5. Bidirectional Test

iperf3 -c 127.0.0.1 --bidir

- Tests simultaneous two-way communication

Observations:

- High bandwidth values were observed during TCP testing.

- UDP test showed low jitter and zero packet loss.

- Parallel streams increased overall throughput.

- Bidirectional mode confirmed simultaneous data transmission.

.